|  | ELEC 205 Assignment 3 Sequential Circuit Design |
| --- | --- |
|  | Deadline: May 11, 11:59 PM |
|  | Name and Surname: Ahmet Buğra Ertürk |
|  | ID: 86877 |

Task 1

I draw the diagram in Figure 1 with 5 states (A/00, B/00, C/00, D/10, E/01). In these states the most significant bit shows Z1, and the least significant bit shows Z0.

Figure 1

Task 2

Because we have 5 states, we need at least 3 binary digits to represent states. I represented the states with the binary codes in Table 1. Unused binary codes are treated as don’t care condition (x).

| State | Binary Code (S2S1S0) |
| --- | --- |
| A | 000 |
| B | 001 |
| C | 010 |
| D | 011 |
| E | 100 |

Table 1

Task 3

The state transition table is in Table 2 with unused state binary codes as don’t care conditions.

| Present State (S2S1S0) | Input (X) | Next State | Output (Z1Z0) |
| --- | --- | --- | --- |
| A (000) | 0 | B (001) | 00 |
| A (000) | 1 | A (000) | 00 |
| B (001) | 0 | B (001) | 00 |
| B (001) | 1 | C (010) | 00 |
| C (010) | 0 | D (011) | 00 |
| C (010) | 1 | E (100) | 00 |
| D (011) | 0 | B (001) | 10 |
| D (011) | 1 | A (000) | 10 |
| E (100) | 0 | B (001) | 01 |
| E (100) | 1 | A (000) | 01 |
| x (101) | x | x | xx |
| x (110) | x | x | xx |
| x (111) | x | x | xx |

Table 2

Task 4

To see Boolean expressions of state binary codes and outputs explicitly, I created K-Maps below.

- S0+ = m(0,2, 4, 6, 8),  d(10,11,12,13,14,15)

Figure 2

  - S0+ = X’

- S1+ = m(3, 4),  d(10,11,12,13,14,15)

Figure 3

  - S1+ = S0’S1X’ + S0S1’X = NAND (NAND (S0’, S1, X’), NAND (S0, S1’, X))

- S2+ = m(5),  d(10,11,12,13,14,15)

Figure 4

  - S2+ = S0’S1X = NAND (NAND (S0’, S1, X), NAND (S0’, S1, X))

- Z0 = m(8, 9),  d(10,11,12,13,14,15)

Figure 5

  - Z0 = S2

- Z1 = m(6, 7),  d(10,11,12,13,14,15)

Figure 6

  - Z1 = S0S1 = NAND (NAND (S0, S1), NAND (S0, S1))

These K-Maps help us to generate explicit Boolean expressions of state binary codes and outputs using only NAND gates. Now we can draw the circuit in Task 5 using D flip-flops to represent each binary code of states and NAND gates as the expressions that we found in this task.

Task 5

I created the circuit in Figure 7 according to expression which I found in Task 4.

Figure 7

Task 6

The timing diagram contains 20 consecutive clock cycles with 3 (0 →1 →0) patterns and 3 (0 →1 →1) patterns as you can see in the diagram. Because of the structure of the D flip-flop in Logisim, which is Edge-Triggered, our states’ values changes only when clock is 1 since slave of the D flip-flop is clock value 1. At the same time master of the D flip-flop is clock value 0, so when we determine the pattern, we have to check the X values when the clock is 0.

Figure 8

When we check the timing diagram in Figure 8 according to my explanation in previous paragraph, output of the timing diagram seems correct.